| Supervisor (print): | Assessment Date(s): |
| --- | --- |
| Signature: | Location(s) posted: |

| # Hazards | Task: hands-on work or being within reach(a) of potential hazards of described activity/items: | Minimum Requirements |
| --- | --- | --- |
| Skin/eye damage, poisoning, inhalation of vapor or aerosol | Volume > 10 mL any unshielded(b) corrosive(c) liquids, organic liquids or liquid mixtures, or toxic(d) inorganic liquids/mixtures | Splash goggles, chemical resistant gloves(e), lab coat, skin cover to knees/elbows/throat, closed shoes with socks. Work in hood(f). Shower and eyewash must be available in work area. |
|  | Volume > 1 L | Same, but cover to ankles/wrists/throat |
|  | Volume > 5 L | Add face shield covering chin |
| Cataracts, flash burns to cornea | Work with infrared emitting equipment (glass blowing) | Appropriate shaded goggles Lab coat, closed shoe, pants |
| Conjunctivitis, corneal damage, erythema | Arc/TIG welding | Appropriate shaded goggles Working gloves |
| Skin/limb injury | Machine operation activities likely to catch clothing, hair, or jewelry | Bind vulnerable clothing/hair, remove jewelry |
| Eye impact | Metalworking, woodworking, other operations likely to throw particles | Safety glasses No loose clothing or jewelry |
| Head impact | Working or walking in area having potential of falling tools, equipment, or stored items | Hard hat |
| Skin/eye damage | Cryogenic liquids | Splash goggles, skin cover to elbows/knees/throat, closed shoe easily removed, socks. Cryogloves for dispensing. |
|  | Volume > 1 L | Skin cover to throat/wrists/ankles |
| Skin/eye damage, asphyxiation, body injury, frostbite, | Transport of liquid nitrogen in hallways and elevators | See cryogenic liquids; also all wheeled vessels or carts must restrain Dewar and have wheels large enough to safely traverse elevator door and scales gap. |
|  | Self pressurizing vessels weighing > 100 lb gross | Plus skin cover to wrists/throat/ankles, always position blow-off valve away from body |
|  | Dispensing from main bulk tank | All above and hearing protection |
| Frostbite, eye impact | Dry ice, very cold frozen solids. | Safety glasses, insulated gloves, skin cover to elbows/knees/throat, closed shoe w/ socks |
| Skin/eye damage | Hot liquid (rxn mixture, water bath, oil bath, autoclave, still...) | Splash goggles, insulated gloves, skin cover to knees/elbows/throat, closed shoe w/ socks |
|  | Volume > 1 L | Skin cover to throat/wrists/ankles, emergency shower available in work area |
| Eye damage, Erythema | UV radiation | UV blocking goggles, skin cover on all potentially exposed areas |
|  | Potential face UV exposure | UV face shield |

| Hazards | Task: (hands-on work or being within reach(a) of potential hazards of described activity/items | Minimum Requirements |
| --- | --- | --- |
| Skin/eye damage | Laser radiation | Goggles appropriate to beam parameters, closed shoe, no jewelry/reflective items |
|  | Class 3b and 4 lasers | Skin cover on all potentially exposed areas |
| Infectious disease | Human blood, cells, tissue, body fluids or materials derived from same | Safety glasses, "exam" gloves, skin cover on all potentially exposed areas, shoes/socks, work at Biosafety Level II. |
|  | Liquid with vol > 1 mL | Same, but splash goggles, skin cover to throat/wrists/ankles |
| Skin/eye damage, poisoning, inhalation of airborne dust | Hazardous solids | Safety glasses, goggles for large quantities, chemical resistant gloves, skin cover to elbows/knees/throat, closed shoes/socks |
|  | > 100 g any hazardous solid, or > 1 g "chemical requiring designated area," (list at REM web site(g)) or High potential for airborne particles | Same, except skin cover to wrists/ankles, and only work in hood |
| Cell damage, area contamination | Radioactive materials | Shielding and badging requirements prescribed in specific isotope SOP, use all appropriate chemical and/or biological safety personal protection |

## NOTES

(a)	Being within reach of potential hazards: "within reach" varies widely depending on scale and conditions of work and will be judged by affected staff in each room.

(b)	Unshielded: not behind a drawn hood sash or blast shield.

(c)	Corrosive: pH  12 or pH  2.5

(d)	Toxic: having any poisonous or irritating effects to human tissue or human health.

(e)	Chemical resistant gloves: glove thickness, length, and material must be chosen carefully and will be specific to the chemicals/mixtures used and the process conditions.

(f)	Hood: 100% exhaust to outside, current approval for "all work" and functioning properly.

(g)	Chemicals requiring designated areas: full list is at http://www.purdue.edu//rem/home/booklets/crdalist.pdf