OCTOBER 1, 2013								    	    www.ISRISafety.org

FALL PROTECTION QUIZ

- How high above the ground or walking level can you get before fall protection is required?

- What must you do before putting on a full body harness or lanyard?

- Name 2 types of fall protection devices.

- Besides a toprail and midrail, what must a guardrail have?

- A full body harness and lanyard are required in a forklift manbasket: True or False?

- A forklift manbasket does not need to be secured to the mast if the forklift is equipped with fork spredders: True or False?

- After a fall, a shock-absorbing lanyard that has been deployed may be re-used as long as it passes inspection: True or False?

- A full body harness and lanyard are required in an aerial lift: True or False?

- Good housekeeping practices are an important part of preventing falls: True or False?

- How many steps can you have before you need handrails?

ANSWERS:

- Four feet.

- Inspect it. Discard if any defects are found.

- Body harness, lanyard, handrails.

- A four-inch toeboard.

- True.

- False.

- False. It must be discarded once deployed.

- True.

- True.

- Four.